TECHNICAL RIDER & GENERAL RIDER

Kirsti Huke

PA:

•    High quality sound system of sufficient size. Must be completely noiseless and capable of delivering undistorted sound all over the venue.

•    High quality mixer, preferably 24 ch. (depends on size of venue)

•    4 separate monitor mixes (separate multiband EQ on all outs)

ch     instrument        suggested mic.                      mic. stand

1       bass drum         M88 / VM44                          lo

2       Snare                 SM57                                     lo

3       OH L                 414 / VM44                           hi

4       OH R                 414 / VM44                           hi

5       Bass                   Active DI                                -

6       Bass                   Active DI  *                            -

7       Piano lo             414 / good condencer            hi

8       Piano lo             414 / good condencer            hi

9       Piano                 SM58                                      hi

10     Vocal                 SM58                                      hi

* Bass player brings his own AMT bass microphone with preamp with XLR line out.

BACKLINE:

Grand piano:    Min. 6 foot Grand piano of high quality, tuned the same day as the concert. Adjustable “soft” piano stool. The piano tuner must be available if needed.

Drums:    1 Snare drum 14" X 4,5"

1 rack tom 12" X 10"

1 floor tom (with legs) 14" X 14"

1 bass drum 18" X 14"

Preferable: Ludwig, Gretch, Ayotte, 70´s Premier or Yamaha.

All heads (top and bottom) must be white-coated Remo Ambassador or Aquarius American Vintage.

Hardware:

1 snare drum stands

3 cymbal stands

1 hi-hat stand

1 bass drum pedal

1 carpet (2 x 2 m)

Preferable: Tama, Premier or Yamaha. Cymbal and hi-hat stands should be single legged and straight without "boom" and must be in perfect condition with felt, screws, rubber ring, hi-hat clutch:

Bass:        1 bass amp. (Polytone, Ampeg, Acoustic Image, or Gallien Krüger)

Lights:        Sufficient

The organiser will also provide/see to that:

•    The sound system and backline must all be rigged/connected and checked before the band arrives for sound check, i.e. ready for immediate use.

•    2 persons available for helping with carriage of instruments/equipment at airport/train station etc, and at venue when arriving for sound check. Same for departure.

•    A sound engineer who knows the sound system and venue, and who is present at get-in, during sound check and concert (the band travels without a sound technician).

•    Dressing room of sufficient size with min. 4 chairs, table and toilet near by. Minimum 20 celcius

Catering etc:

•    Dinner (a-la-carte menu) at sufficient restaurant.

•    Mineral water and 5 towels at stage during concert

•    For sound check and in dressing room after concert: Assorted juice and mineral water, coffee and baguettes. Assorted fruits and snacks. Beers and 2 bottles of adequate wine after concert.

And finally:

•    The band members are allowed 1 guest each (4 in total) for free entrance at the concert

•    The band would like the organiser to introduce the band from stage before concert

•    One person who can sell CDs in the break/after the concert

NB! This rider is part of the contract between the band and the local organiser that is read, understood and signed by both parts.